USB to Serial Bridge Cable User’s Manual

Introduction:

The USB to Serial Bridge Cable is your fast solution to utilize the peripheral with serial port in an easy-to-use environment such as plug-n-play and hot swap function. This Bridge Cable provides ideal connections to Modems, PDA, Digital Camera, Label printer or ISDN terminal adapter with over 1Mbps data transfer rate.

- Features:

- Full compliance with the USB specification Rev 1.1

- Support the RS232 Serial interface

- Over 1Mbps data transfer rate

- Support remote wake-up and power management

- Single chip (ASIC) USB to Serial communication

- System Requirements:

- Intel-compatible 486DX-66 MHz CPU or higher

- One standard USB port (4-pin) and has the symbol

- Either the following operating systems

- Supports Windows 98, 98SE, ME and 2000

- Supports Mac OS 8.5 and above

- Driver Installation:

Follow the steps below on installing the USB to Serial Bridge Cable for the first time:

- Power on both computers where you will connect the USB to Serial Bridge Cable and make sure that the USB port is enabled and working properly.

- Plug in the USB to Bridge Cable into the USB port and Windows will detect an USB Device and run the Add New Hardware Wizard to assist you in setting up the new USB Composite Device.

- Insert the USB to Serial Bridge Cable driver diskette into the floppy drive and click Next to continue:

- Select Search for the best driver for your device and click next.

- Select Specify a location and click Browse. Change the folder of your floppy drive (ie: A:\ WIN98) and click OK.

- Double check the directory that Windows prompts. Click Next.

- Windows will detect the driver (USB2SER.inf) and shows the USB to Serial Bridge Cable. Click NEXT to continue until installation is complete.

- Click Next to continue and let Windows copy the needed files to your hard disk. Click Finish while installation is complete.

- After installing, the System will generate an additional COM Port, USB to Serial Port (COM3) for the connection to RS232 Serial Device.

- Setting Up the RS232 Serial Device:

Follow the steps below on how to connect your RS232 Serial Device (ie: Modem) to the USB port of your PC:

- Turn off your Modem. Plug the D-Sub 9 connector of the USB to Serial Bridge Cable into your Modem. If you have a Serial cable connected to it, remove the cable and replace it with the USB to Serial Bridge Cable. Turn on the Modem afterwards.

- Turn on your computer and plug in the USB connector of the USB to Serial Bridge Cable into the USB port.

- If you have already installed a Modem device before, click Start, Settings, and Modems. Open Modems Properties, select the Modem, and click Properties. In General tab, please change the Port value to USB to Serial Port (COM3).

- After finishing these processes, you can use USB to Serial Bridge Cable to connect RS232 Serial 56K FAX Modem and execute the Dial-up fuction to connect Internet, and send the FAX with the FAX software (ie: Microsoft FAX) properly.

- Driver Uninstallation:

You can uninstall the USB to Serial Bridge Cable driver by executing Uninstall Program: uninstall.exe in the driver diskette and click the Remove button to accomplish the uninstallation.

- Packing List:

- USB to Serial Bridge Cable

- One Floppy Diskette with installation Driver

- This User’s Manual

Update: 20010104